If MAC Address was error, please refer attachment tool to fix it.

Please refer below steps to fix MAC address.

STEP1:  Find motherboard’s MAC address on rear panel and write it down.

(You could refer attachment image file to find motherboard’s MAC address NO.)

STEP2:  Unzip attachment zip file to a bootable Floppy diskette (bootable USB pen driver).

STEP3:  Boot into pure DOS environment

SETP4   Enter the file folder, key in “mac c [mac ID], press ENTER.

(EX:  A:\MAC> mac c [mac ID])

Monitor will print successful message on screen after updating MAC address successfully.

STEP5:  Reboot your system

- If you did not have floppy drive, you could use bootable USB pen driver to fix it.

- Please press F11 (Boot menu) at PC boot up and choose USB pen driver to boot into pure DOS

***************************************************************************

请您使用附件中更新主板MAC的软件输入您的MAC地址

请依下列步骤操作软件输入您的MAC地址:

STEP1 在主板输出/入插座面板上找到您主板MAC地址号码记下来

STEP2 制作一片开机磁盘，同时也将附件中的压缩档解压缩到这一片可开机的软盘中(或是可开机的U盘)

STEP3 开启计算机进入纯DOS的环境

STEP4 进入解压缩后的文档资料夹中，键入mac c [ mac ID ] 然后按ENTER

(输入更新成功后，屏幕上会显示更新成功的信息画面)

STEP5 重起您的计算器

*您可依循附件中图档的位置，找到您主板的MAC地址号码

*.如果您没有软驱，请使用可开机的U盘来修改MAC地址号码

*请再开机时按F11(启动菜单)，选择您的U盘进入纯DOS环境